[image: image1.jpg]l&?‘!l





Ecosure 4400 Litre Rainwater Harvesting Kit Installation guide

[image: image1.jpg]
The arrangement of the 1100 Litre underground tanks is up to you.  We display these in rows of two.  They can be arranged in other ways to suite your purpose. 

Pipe Work

The fittings to be used on the underground tanks are 4” (110mm) which is not supplied.
Step 1

Dig out where you want your tanks to be. The tank holes should be approx 1250mm deep and 1850mm in diameter. Please allow 300mm gap between each tank and a gap of 150mm around the perimeter of each tank to allow for back filling.


Also at this stage you should dig out the trenches for your pipe work and in cylindrical filter.
Step 2

a) At the bottom of each tank hole, pour a layer of concrete (50mm) to create a flat base. Allow the concrete to dry and slide the tanks in to each hole. Fill each tank with a little water to keep the tanks flat and underground.  If your ground type requires it, you may need to fill up past the outer ring with concrete. 
b) Choose a suitable space in a building for your pump to sit. Make sure it has 240V electricity nearby. Connect the small filter to the pump (as shown in the Centrifugal pump instructions).
c) Now take the BLUE suction pipe out to the underground water tanks (This can run alongside the 4” pipe).  This only needs to be connected to one of the water tanks. 
Step 3

[image: image2.jpg]


From the buildings down pipe, connect your 4” (110mm) pipe work to the cylindrical filter, then connect the 4” (110mm) pipe work to the underground tanks.


Back fill the area surrounding the tank with pea shingle (preferably).  

Step 4

. 
a) Once all underground pipe work is connected back fill all open trenches.
b) From top of Centrifugal pump connect the destination pipe (e.g. Sink, Garden Hose etc..)
Please note: These instructions are a guide to the installation as ground types and environment can change the installation. If unsure please contact your local builder. 





From Down Pipe





�INCLUDEPICTURE "../../Local%20Settings/Temp/Save%20For%20Web/Untitled-4_001.tmp/Untitled-4.jpg" \* MERGEFORMATINET ���





To other tanks








2
www.ecosure.co.uk
Sales: 0871 200 2082

